SIMATS ENGINEERING
ADVANCED WIRELESS COMMUNICATION SYSTEMS

ASSIGNMENT – 1

DESIGN AND OPTIMIZATION OF AN INTEGRATED
5G/6G-READY COMMUNICATION NETWORK

Submitted by
Name: Akshaya R
Register Number: 192512399
Course Name: 5G/6G Communication SysteM

# TABLE OF CONTENTS

| S. No. | Contents |
| --- | --- |
| 1 | Introduction |
| 2 | Stage 1 – Engineering Problem and Requirements |
| 3 | Stage 2 – Spectrum, Radio Access and Cellular Planning |
| 4 | Stage 3 – Capacity, Latency and Reliability Models |
| 5 | Stage 4 – Network Architecture Evaluation |
| 6 | Stage 5 – Integrated 5G/6G Network Design |
| 7 | Stage 6 – Design Validation and Deployment Decision |
| 8 | Overall Conclusion |
| 9 | References |

# INTRODUCTION

Wireless communication has developed from first-generation analogue mobile systems to advanced 5G networks. Future 6G systems are expected to provide intelligent, highly reliable and extremely high-capacity communication. The proposed engineering design considers an integrated 5G/6G-ready communication network across Tamil Nadu, Karnataka, Kerala, Maharashtra, Gujarat and Uttar Pradesh.

The network is planned to support enhanced mobile broadband, autonomous transportation, remote healthcare, industrial automation, massive Internet of Things (IoT) connectivity and emergency communication. Since the six states contain urban, suburban, rural, industrial and transport environments, a single network configuration cannot meet every requirement. Therefore, spectrum bands, cell sizes and network services are selected according to regional needs.

The report follows the six stages specified in the assignment: requirements analysis, spectrum and radio-access planning, quantitative modelling, architecture evaluation, integrated network design, and validation.

# STAGE 1 – ANALYZE THE ENGINEERING PROBLEM AND DEFINE REQUIREMENTS (10 MARKS)

## 1.1 Communication Requirements

The main objective is to design a network that provides sufficient coverage, capacity, low latency, reliability and security at a reasonable deployment and operating cost. The network must support both high-data-rate services and low-power connections for a large number of IoT devices.

High-speed and stable broadband connectivity for mobile users.

Low latency and high reliability for autonomous vehicles and industrial control.

Wide-area coverage for rural communities and emergency services.

Connectivity for a large number of sensors and smart devices.

Secure user authentication and protection of transmitted data.

Efficient energy use and affordable network deployment.

## 1.2 Application Classification

| Application | Service Category | Main Requirement |
| --- | --- | --- |
| HD video and mobile broadband | eMBB | High data rate and capacity |
| Autonomous transportation | URLLC | Very low latency and high reliability |
| Remote healthcare | URLLC | Reliable, low-delay communication |
| Industrial automation | URLLC | Fast and dependable control |
| Smart sensors and meters | mMTC | Large device density and low power |
| Emergency communication | Public safety / URLLC | Priority and resilient connectivity |

## 1.3 State-Wise Requirements Matrix

| State | Major communication environment | Main challenge | Proposed direction |
| --- | --- | --- | --- |
| Tamil Nadu | Cities, manufacturing and transport | Industrial traffic and urban congestion | Mid-band macro cells with small cells in dense locations |
| Karnataka | IT hubs, cities and technology parks | High broadband and enterprise demand | Mid-band, massive MIMO and selected mmWave hotspots |
| Kerala | Coastal, hilly and rural regions | Coverage and terrain-related signal variation | Sub-1 GHz macro coverage with mid-band in towns |
| Maharashtra | Large cities, industry and transport | Very high user density and traffic | Mid-band densification and mmWave hotspots |
| Gujarat | Industrial zones, ports and cities | Reliable industrial connectivity | Mid-band and private 5G networks where required |
| Uttar Pradesh | Large urban and rural population | Wide-area coverage and capacity variation | Sub-1 GHz macro coverage with mid-band in populated areas |

## 1.4 Engineering Constraints

Spectrum availability and licensing restrictions.

Propagation loss, building blockage and terrain conditions.

Interference between neighbouring cells and networks.

Cost of towers, base stations, fibre and backhaul.

Power consumption and environmental impact.

Cybersecurity, privacy and protection of critical services.

Service continuity during congestion, disasters and equipment failures.

## 1.5 Performance Indicators and Design Objectives

| Performance indicator | Proposed planning target |
| --- | --- |
| Broadband user data rate | 100 Mbps or higher where coverage and load permit |
| Critical application latency | Below 10 ms as a planning target |
| Critical-service reliability | Up to 99.999% target, subject to end-to-end design |
| Traffic-channel blocking probability | Not more than 2% |
| General packet loss | Below 1% planning target |
| Cell utilization | Target below 70% under normal planning load |

These values are proposed engineering targets for the sample design. Actual performance must be confirmed through field measurements, licensed spectrum details and network simulation.

## 1.6 Constrained Optimization Formulation

The design can be treated as a constrained optimization problem. The design variables include spectrum band, cell radius, number of base stations, antenna configuration, transmit power, backhaul capacity and resource-allocation policy. The objectives are to maximize coverage, throughput and reliability while minimizing latency, interference, energy consumption and total cost. Constraints include available spectrum, site availability, budget, regulatory limits and required service performance.

Design Decision 1: Capacity and reliability are prioritized for critical services, while broad coverage and cost efficiency are emphasized in rural regions. Conflicting requirements are balanced through different cell types and spectrum bands.

# STAGE 2 – ANALYZE SPECTRUM, RADIO ACCESS AND CELLULAR NETWORK PLANNING (15 MARKS)

## 2.1 Spectrum Selection

| Spectrum range | Characteristics | Suitable applications |
| --- | --- | --- |
| Sub-1 GHz | Longer range and better building penetration; comparatively limited bandwidth | Rural coverage, IoT and emergency communication |
| Mid-band (1–6 GHz) | Balance between coverage and capacity | Urban, suburban, industrial and transport areas |
| mmWave (>24 GHz) | Very large bandwidth potential; shorter range and blockage sensitivity | Dense urban hotspots and fixed high-throughput links |

Sub-1 GHz is suitable for wide-area coverage. Mid-band is the main capacity layer for most urban, suburban and industrial deployments. mmWave should be used selectively in high-traffic locations because it requires denser infrastructure and careful line-of-sight planning.

## 2.2 Cellular Deployment Options

| Cell type | Coverage role | Typical use |
| --- | --- | --- |
| Macro cell | Large-area coverage | Rural areas, highways and general outdoor coverage |
| Micro cell | Medium-sized coverage area | Busy streets and commercial areas |
| Pico cell | Small coverage area | Indoor public spaces and offices |
| Femto cell | Very small indoor coverage | Homes and small offices |

## 2.3 Radio Access Technologies

5G New Radio (NR) uses OFDMA-based radio access and supports flexible resource allocation. Massive MIMO uses multiple transmit and receive antennas to serve users efficiently. Beamforming focuses radio energy towards intended users, improving signal quality and reducing unnecessary interference. Heterogeneous networks combine macro cells and small cells to improve coverage and capacity.

## 2.4 Frequency Reuse Calculation

Frequency reuse allows the same frequency resources to be used in geographically separated cells. The approximate reuse distance is:

D = R × √(3N)

Assume a cell radius R = 1 km and cluster size N = 7.

D = 1 × √(3 × 7) = √21 = 4.58 km

Therefore, the approximate co-channel reuse distance is 4.58 km under the stated idealized hexagonal-cell assumption.

## 2.5 Capacity Enhancement and Interference Management

Cell splitting: divides a large cell into smaller cells to increase capacity.

Sectoring: uses directional antennas to reduce interference and improve reuse.

Beamforming: directs transmission towards the intended user.

Power control: adjusts transmit power to reduce unnecessary interference.

Interference coordination: coordinates resource use between neighbouring cells.

Careful frequency planning: reduces co-channel and adjacent-channel interference.

## 2.6 State-Wise Spectrum and Radio-Access Plan

| State | Recommended spectrum | Radio-access strategy |
| --- | --- | --- |
| Tamil Nadu | Sub-1 GHz + mid-band; mmWave hotspots | Macro coverage and small cells near industries/cities |
| Karnataka | Mid-band; mmWave hotspots | Massive MIMO macro cells and dense small cells |
| Kerala | Sub-1 GHz + mid-band | Wide-area macro coverage and targeted town capacity |
| Maharashtra | Mid-band + mmWave; sub-1 GHz coverage | Macro layer with dense small cells in high-traffic zones |
| Gujarat | Sub-1 GHz + mid-band | Macro cells and industrial/private network deployments |
| Uttar Pradesh | Sub-1 GHz + mid-band | Macro-led coverage with capacity cells in cities |

Design Decision 2: A mixed spectrum and heterogeneous-cell strategy is recommended. A uniform band or cell type would be inefficient because population density, terrain, traffic and application requirements differ across the six states.

# STAGE 3 – DEVELOP QUANTITATIVE CAPACITY, LATENCY AND RELIABILITY MODELS (20 MARKS)

## 3.1 Sample Traffic Demand

The assignment does not provide a complete numerical subscriber and traffic dataset. The following values are therefore assumed only to demonstrate the engineering calculations for a sample planning area.

| Application | Number of users/devices | Assumed rate | Aggregate traffic |
| --- | --- | --- | --- |
| HD video | 1,000 users | 8 Mbps | 8,000 Mbps |
| Autonomous vehicles | 500 vehicles | 2 Mbps | 1,000 Mbps |
| Industrial IoT | 20,000 devices | 0.1 Mbps | 2,000 Mbps |
| Total | — | — | 11,000 Mbps (11 Gbps) |

Total traffic = 8,000 + 1,000 + 2,000 = 11,000 Mbps = 11 Gbps.

## 3.2 Practical Cell Capacity

The approximate data-rate relationship is R = B × η, where B is bandwidth and η is spectral efficiency.

Assume bandwidth B = 100 MHz and spectral efficiency η = 4 bits/s/Hz.

Theoretical rate = 100 × 10⁶ × 4 = 400 Mbps.

Assuming 70% practical utilization, practical cell capacity = 400 × 0.70 = 280 Mbps.

## 3.3 Minimum Number of Cells

Number of cells = Total traffic demand / Practical capacity per cell.

N = 11,000 / 280 = 39.29. Therefore, at least 40 cells are required for this sample traffic estimate.

This is a simplified capacity estimate. Real deployment requires additional allowance for uneven traffic distribution, control overhead, interference, redundancy, mobility and busy-hour variation.

## 3.4 Shannon Capacity

The Shannon capacity equation is C = B log₂(1 + SNR), where C is theoretical channel capacity in bits per second, B is bandwidth in hertz and SNR is the linear signal-to-noise ratio. The equation gives an upper theoretical limit; practical throughput is lower due to interference, protocol overhead and implementation limitations.

## 3.5 Latency and Reliability

| Application | Proposed latency target | Reason |
| --- | --- | --- |
| HD video streaming | Below 50 ms | Smooth interactive media experience |
| Autonomous vehicles | Below 10 ms | Fast exchange of safety-related information |
| Industrial automation | 1–10 ms, depending on task | Timely control and monitoring |
| Remote healthcare | Below 10 ms for critical interactive control | Responsive communication |

End-to-end latency includes radio transmission, transport/backhaul, core-network processing and application processing. Placing edge-computing resources near vehicles, hospitals and industrial sites can reduce the distance travelled by time-sensitive data. Reliability also depends on redundant links, coverage overlap, backup power and resilient core services.

## 3.6 Erlang B Traffic-Channel Calculation

The Erlang B model estimates the probability that a call or traffic request is blocked when all traffic channels are occupied. The assignment asks for a target blocking probability of 2%. Since the supplied PDF does not contain actual subscriber count, busy-hour traffic per subscriber or available channel values, the following sample assumptions are used.

| Input parameter | Assumed value |
| --- | --- |
| Number of subscribers | 500 |
| Busy-hour traffic per subscriber | 0.02 Erlang |
| Offered traffic A | 500 × 0.02 = 10 Erlangs |
| Available traffic channels | 17 |
| Target blocking probability | 2% |

Erlang B formula:

B(A,N) = (Aᴺ / N!) / Σ(k=0 to N) (Aᵏ / k!)

For offered traffic A = 10 Erlangs and N = 17 channels, the blocking probability is approximately 1.3%.

Since 1.3% is less than the target 2%, 17 channels satisfy the target for these assumed inputs. Guard bands, duplexing overhead and other practical spectrum overheads are not included in this idealized traffic-channel example; actual spectrum planning must account for them.

## 3.7 Limitations

The estimates use assumed traffic rates, spectral efficiency and subscriber data. Actual values depend on measured busy-hour demand, licensed bandwidth, radio conditions, device capabilities, traffic mix and operator design. Detailed simulation and field measurements are required before deployment.

Design Decision 3: The sample calculation indicates that capacity may be a key constraint in dense urban and industrial zones. Small-cell deployment, additional spectrum, improved spectral efficiency and edge computing are suitable interventions. In rural regions, coverage and backhaul availability may be the dominant constraints.

# STAGE 4 – EVALUATE NETWORK ARCHITECTURES AND SELECT AN OPTIMAL DESIGN (15 MARKS)

## 4.1 Alternative A – Macro-Cell-Focused Architecture

Architecture A uses sub-1 GHz and mid-band macro cells to provide broad coverage with fewer sites. It is comparatively economical and suitable for rural and low-density areas. However, it may not provide sufficient capacity in crowded urban locations or very low latency for all applications.

## 4.2 Alternative B – Heterogeneous Network

Architecture B combines macro cells, micro cells, pico cells, mid-band, selected mmWave hotspots and edge computing. It can provide high capacity and low latency in dense locations, but requires more sites, careful interference coordination, higher investment and more complex operation.

## 4.3 Weighted Decision Matrix

| Criterion | Weight | A score (1–5) | B score (1–5) |
| --- | --- | --- | --- |
| Coverage | 20% | 5 | 4 |
| Capacity | 25% | 3 | 5 |
| Latency | 20% | 3 | 5 |
| Reliability | 15% | 4 | 5 |
| Cost | 10% | 5 | 3 |
| Scalability | 10% | 3 | 5 |
| Weighted total (out of 5) | 100% | 3.80 | 4.60 |

Weighted score = Σ (criterion weight × score). The weights total 100%. Architecture A score = 0.20(5) + 0.25(3) + 0.20(3) + 0.15(4) + 0.10(5) + 0.10(3) = 3.80. Architecture B score = 0.20(4) + 0.25(5) + 0.20(5) + 0.15(5) + 0.10(3) + 0.10(5) = 4.60.

The scores are engineering judgments for this proposed design, not measured field results.

## 4.4 Network Slicing and Edge Computing

Network slicing creates separate logical networks over a shared physical infrastructure. An eMBB slice serves broadband and video traffic; a URLLC slice serves vehicle and industrial-control traffic; an mMTC slice serves large numbers of IoT devices; and a public-safety slice provides priority communication during emergencies.

Edge-computing nodes should be placed near major transport corridors, industrial clusters, hospitals and dense urban areas. Processing data locally reduces the need to send every time-sensitive request to a distant cloud data centre.

## 4.5 Preferred Architecture

Architecture B is selected because its weighted score is higher and it better supports high capacity, low latency and future expansion. Architecture A remains useful as the basic coverage layer in rural and low-density areas. The final design therefore uses a hybrid approach: macro-led coverage everywhere, with small cells and edge computing added where traffic or latency requirements justify the cost.

Design Decision 4: If rural coverage becomes the dominant requirement, more sub-1 GHz macro cells and improved backhaul are prioritized. If urban video throughput dominates, mid-band capacity and mmWave small cells are increased. If vehicle reliability dominates, transport-corridor coverage, redundant backhaul and edge processing receive priority.

# STAGE 5 – CREATE THE INTEGRATED 5G/6G NETWORK DESIGN (20 MARKS)

## 5.1 Proposed Integrated Architecture

The proposed network contains user equipment, 5G New Radio base stations (gNodeBs), the Radio Access Network (RAN), a 5G Core, edge-computing nodes, backhaul and cloud services. User equipment communicates with a nearby gNodeB. The RAN forwards traffic to the 5G Core through transport connectivity. The core manages access, sessions, policy and user-data forwarding. Latency-sensitive applications can be routed to nearby edge resources.

USER EQUIPMENT (phones, vehicles, sensors, medical devices) ↓ 5G NR RADIO ACCESS NETWORK – gNodeBs / macro cells / small cells ↓ TRANSPORT / BACKHAUL ↓ 5G CORE NETWORK AMF ¦ SMF ¦ UPF ¦ AUSF ¦ UDM ¦ PCF ↙ ↘ EDGE COMPUTING CLOUD / INTERNET ↓ Applications: transport, healthcare, industry, IoT, emergency

## 5.2 5G Core Network Functions

| Core function | Main responsibility |
| --- | --- |
| AMF – Access and Mobility Management Function | Manages registration, access and mobility. |
| SMF – Session Management Function | Establishes and manages user sessions. |
| UPF – User Plane Function | Forwards user data between RAN and data networks. |
| AUSF – Authentication Server Function | Supports subscriber authentication. |
| UDM – Unified Data Management | Stores and manages subscriber information. |
| PCF – Policy Control Function | Provides policy and service-control rules. |

## 5.3 Logical Network Slices

| Slice | Users/services | Priority and design purpose |
| --- | --- | --- |
| eMBB | Mobile broadband and HD video | High throughput and capacity |
| URLLC | Autonomous vehicles and industrial control | Low latency and high reliability |
| mMTC | Smart meters and sensor networks | Large device density and efficient signalling |
| Public safety | Emergency teams and disaster response | Priority access and resilient connectivity |

## 5.4 AI-Based Network Optimization

Traffic prediction: estimates busy-hour demand using historical network measurements.

Dynamic resource allocation: assigns radio resources according to service demand.

Interference management: identifies high-interference cells and supports coordinated adjustments.

Fault detection: identifies abnormal network behaviour and supports faster maintenance.

Energy optimization: places lightly loaded cells into suitable energy-saving states while preserving coverage.

## 5.5 Mobility Management

For vehicles moving along highways and transport corridors, neighbouring cells should have overlapping coverage and coordinated handover settings. The network monitors signal quality and transfers the connection to a suitable target cell as the vehicle moves. Redundant backhaul and edge nodes along important corridors help maintain service continuity.

## 5.6 Cybersecurity and Resilience

Use secure subscriber authentication and encryption.

Apply role-based access control and secure configuration management.

Monitor traffic for anomalies and possible attacks.

Use redundant core functions and diverse backhaul routes for critical locations.

Provide backup power for important base stations and emergency sites.

Maintain incident response, recovery and regular security-update procedures.

## 5.7 Sustainability and Future 6G Concepts

Energy efficiency can be improved through cell sleep modes, dynamic power control, efficient antennas, renewable-energy supplies where practical and infrastructure sharing. Future 6G research concepts include AI-native networking, integrated sensing and communication (ISAC), and potential terahertz communications. These are future-facing concepts and should be distinguished from currently deployable 5G capabilities.

## 5.8 Design Validation Method

The design can be validated using MATLAB, Python or ns-3. The model should represent traffic demand, cell capacity, mobility, spectrum allocation, latency and interference. Results should be compared against the stated performance targets and the baseline architecture. Simulation outputs must be labelled as simulated results and not as field measurements.

Design Decision 5: The state-wise design combines a common 5G core and service framework with different radio-access layers. Sub-1 GHz supports broad coverage, mid-band provides the main capacity layer, and mmWave is used selectively. Edge computing and network slicing support latency-sensitive and critical services. 6G concepts are included as future upgrade provisions.

# STAGE 6 – VALIDATE THE DESIGN AND MAKE THE FINAL DEPLOYMENT DECISION (20 MARKS)

## 6.1 Validation Approach

The principal traffic, capacity, channel and latency estimates should be checked using spreadsheet calculations or a simulation tool. The sample cell calculation gives 40 cells for 11 Gbps of assumed aggregate traffic when each cell provides 280 Mbps practical capacity. The sample Erlang B calculation gives approximately 1.3% blocking for 10 Erlangs offered to 17 channels, meeting the 2% target.

## 6.2 Performance Verification Table

| Requirement | Design response | Evidence / validation method | Status |
| --- | --- | --- | --- |
| Broadband throughput | 100 MHz mid-band example; capacity planning | R = Bη calculation and throughput simulation | Proposed |
| Rural coverage | Sub-1 GHz macro cells | Coverage prediction and field drive test | Proposed |
| Low latency | Edge computing and URLLC slice | End-to-end latency measurement | Proposed |
| Traffic blocking ≤ 2% | Traffic-channel dimensioning | Erlang B sample calculation ≈ 1.3% | Meets sample target |
| IoT connectivity | mMTC slice and scalable access | Device-load simulation | Proposed |
| Security | Authentication, encryption and monitoring | Security audit and penetration testing | Proposed |
| Energy efficiency | Sleep modes and power optimization | Energy-per-bit and site-power monitoring | Proposed |

## 6.3 Scenario 1 – Peak-Hour Congestion

During peak hours, a large number of users may access video, broadband and enterprise services simultaneously. This can increase cell utilization, delay and packet loss. The network should respond by allocating resources dynamically, steering users to less-loaded cells, activating additional small cells where available and prioritizing critical traffic. Capacity expansion should be based on measured busy-hour load.

## 6.4 Scenario 2 – Emergency or Infrastructure Failure

During a flood, cyclone, accident or power failure, some network sites or backhaul links may become unavailable. Emergency slices should receive priority. Backup power, redundant transport routes, portable cells and alternate routing can help restore communication. Critical hospitals, emergency centres and transport control facilities should be included in resilience planning.

## 6.5 Deployment Risks and Limitations

The actual subscriber distribution and busy-hour traffic may differ from the assumed sample values.

Spectrum licensing and availability must be confirmed with the relevant authorities.

Site acquisition, fibre availability and power supply may delay deployment.

mmWave coverage can be affected by buildings, trees, weather-related conditions and blockage.

Cybersecurity threats and equipment failures require continuous monitoring.

The proposed performance targets require field testing and end-to-end verification.

## 6.6 Final Deployment Recommendation

The recommended decision is deployment with modifications and phased validation. The network should first be introduced in selected high-demand urban and industrial locations, followed by suburban, rural and transport-corridor expansion. Each phase should be reviewed using measured coverage, throughput, latency, reliability, energy use and cost.

The preferred heterogeneous architecture offers a flexible balance between wide-area coverage and high-capacity service. Macro cells provide the foundation, while small cells, mmWave hotspots, edge computing and network slicing are added where justified by traffic and application requirements.

# OVERALL CONCLUSION

This report presented an integrated 5G/6G-ready communication network design for Tamil Nadu, Karnataka, Kerala, Maharashtra, Gujarat and Uttar Pradesh. The analysis shows that regional differences in population density, terrain, industry and application demand require a flexible deployment strategy. Sub-1 GHz is suitable for wide-area and rural coverage, mid-band provides a balance of coverage and capacity, and mmWave supports selected high-capacity hotspots.

The quantitative examples estimated 11 Gbps of sample aggregate traffic, a practical cell capacity of 280 Mbps and a requirement of approximately 40 cells. The sample Erlang B calculation showed that 17 channels support 10 Erlangs with an estimated blocking probability of about 1.3%, below the 2% target. These results are based on stated assumptions and must be replaced or refined when actual planning data are available.

A heterogeneous architecture with macro cells, small cells, 5G Core functions, network slicing, edge computing, AI-assisted optimization, cybersecurity and energy-saving measures is recommended. Future 6G concepts such as AI-native networking and integrated sensing and communication can be considered during later upgrades. Final implementation should proceed in phases after simulation, field measurements, spectrum confirmation and economic evaluation.

# REFERENCES

T. S. Rappaport, Wireless Communications: Principles and Practice, 2nd ed., Prentice Hall, 2002.

A. Goldsmith, Wireless Communications, Cambridge University Press, 2005.

3GPP, 5G System Architecture, 3GPP TS 23.501.

3GPP, NR; Physical Layer Procedures for Control, 3GPP TS 38.213.

ITU-R, Minimum Requirements Related to Technical Performance for IMT-2020, Recommendation ITU-R M.2410.

ITU-R, Framework and Overall Objectives of the Future Development of IMT for 2030 and Beyond, Recommendation ITU-R M.2160.